SAMPLE SYSTEM SPECIFICATION - Power Mode Control Signal Contract

Purpose: fake/non-confidential sample input for the Power Mode Test Spec Assistant. This Word file defines signals, value domains, initial values, fail-safe values, and a Japanese excerpt for review.

# 1. Metadata

| Document ID | PM-SYS-SPEC-SAMPLE-001 | Revision | A |
| --- | --- | --- | --- |
| Language | English + Japanese | Scope | Power Mode signal contract |

# 2. System Overview

Power Mode Control receives input requests and vehicle conditions, evaluates transition condition trees, updates internal states, controls hardware outputs, and publishes interface status.

# 3. Glossary

| Term | Meaning | Term | Meaning |
| --- | --- | --- | --- |
| ADM1_ACC | Confirmed manual ACC state | ADM1_OFF | Confirmed manual OFF state |
| Transitioning State | Temporary state while output is applied | T_shutdown | Timer for shutdown request duration |

# 4. Interface Definitions

The assistant should extract signal name, direction, value domain, initial value, and fail-safe value from this table.

| Signal Name | Direction | Sender | Receiver | Length | Initial | Fail-safe | Value Domain / Meaning | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mode_cmd | Input | BCM | PM_CTRL | 2 bits | 0 | 0 | 0=No request; 1=Shutdown request; 2=ACC request; 3=Reserved | Customer command for power mode transition. |
| IGN_SW | Input | IGN_SWITCH | PM_CTRL | 1 bit | 0 | 0 | 0=OFF; 1=ON | Ignition switch status. |
| VehicleSpeed | Input | ABS | PM_CTRL | 16 bits | 0 | 0 | 0..250 km/h; 0=Stopped | Vehicle speed value. |
| Battery_OK | Input | BMS | PM_CTRL | 1 bit | 1 | 0 | 0=Abnormal; 1=Normal | Battery voltage and BMS availability status. |
| DoorLock_STS | Input | BCM | PM_CTRL | 1 bit | 0 | 0 | 0=Unlocked; 1=Locked | Door lock status used as optional shutdown precondition. |
| Mode_STS | Output | PM_CTRL | OEM_IF | 2 bits | 1 | 0 | 0=OFF; 1=ACC; 2=IGON; 3=Reserved | External published power mode status. |
| ACC_Relay | Output | PM_CTRL | RelayDriver | 1 bit | 0 | 0 | 0=OFF; 1=ON | Hardware ACC relay command. |

# 5. Japanese Source Excerpt for Review

Original Japanese requirement excerpt: Mode_cmd が 1 の状態で T_shutdown が 100ms 以上継続した場合、Mode_STS は 0 を出力すること。

Expected AI technical interpretation candidate: Given Mode_cmd = 1; When T_shutdown >= 100ms; Then Mode_STS = 0. Mark as comtor_review_required and engineer_review_required.

# 6. Fail-safe Notes

If Battery_OK = 0 or timeout is detected during shutdown transition, the system shall fall back to ADM1_OFF and publish Mode_STS = 0. Undefined values shall use fail-safe value if no explicit behavior is defined.